| MINIMUM SPECIAL INSPECTOR QUALIFICATIONS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category of Testing and Inspection | Minimum Qualifications (refer to key at end of Table) |  |  |  |
|  | Shop Inspection | Field Testing /Inspection | Review Submittals | Review Testing, Certification, & Lab Reports |
| 1704.2.5 Inspection of Fabricators |  |  |  |  |
| Pre-cast concrete | A, C, E |  |  |  |
| Structural steel construction | C, F, G |  |  |  |
| Wood construction | A, N |  |  |  |
| Cold formed metal construction | A, N |  |  |  |
| 1705.2 Steel Construction |  |  |  |  |
| Welding | C, F, G | C, F, G | A | A |
| High strength bolting, inspection of steel frame joint details |  | A, C | A | A |
| 1705.2.2, 1705.2.3 and 1705.2.4 Steel Construction other than Structural Steel |  |  |  |  |
| Welding | C, F, G | C, F, G | A | A |
| Cold-formed Steel Deck |  | C, F, G | A | A |
| Open-Web Steel Joist and Joist Girders |  | C, F, G | A | A |
| Cold-formed Steel Trusses spanning ≥ 60ft |  | A, C | A | A |
| 1705.3 Concrete Construction |  |  |  |  |
| Reinforcing placement, cast-in-place bolts, concrete and shotcrete placement and curing operations |  | A, C, H |  |  |
| Pre-stressing steel installation |  | A, C, D, E |  |  |
| Erection of pre-cast concrete members |  | A, C, H, Q |  |  |
| Concrete field testing |  | A, C, H, I, J |  |  |
| Review certified mill reports and design mixes |  |  | A |  |
| Verify use of required design mix |  | A, C, H, I, J |  |  |
| Pre-stressed (pre-tensioned) concrete force application | A, C, E |  |  |  |
| Post-tensioned concrete force application |  | A, C, D, H |  |  |
| Review of in-situ concrete strength, prior to stressing of tendons in post-tensioned concrete and prior to removal of shores and forms from beams and structural slabs |  | A, C, D |  |  |
| Reinforcing steel weldability, reinforcing welding, weld filler material |  | C, F, G |  |  |
| Inspection of post-installed anchors in hardened concrete |  | A, C, S |  |  |
| 1705.4 Masonry |  |  |  |  |
| Review f’m prior to construction |  |  | A |  |
| Mortar joint construction, grout protection and placement, materials proportion, type/size/location of reinforcement, structural elements, anchorage, and connectors |  | A, C |  |  |
| Sampling/testing of grout/mortar specimens |  | A, C, K |  |  |
| Observe preparation of masonry prisms for testing of compressive strength of masonry, f’m |  | A, C, K |  |  |
| Inspection of welding of reinforcing steel |  | C, F, G |  |  |
| 1705.5 Wood Construction |  |  |  |  |
| Observe structural panel sheathing, size of framing members, fastener diameter and length, number of fastener lines, and spacing of fastener lines and fasteners for compliance with approved construction documents for the project |  | A, N |  |  |
| Metal-plate-connected wood trusses: verify temporary and permanent truss bracing is installed per approved truss submittal package |  | A, N |  |  |
| (Table continued on next page) |  |  |  |  |

| MINIMUM SPECIAL INSPECTOR QUALIFICATIONS (continued) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Category of Testing and Inspection | Minimum Qualifications (refer to key at end of Table) |  |  |  |  |  |  |
|  | Shop Inspection | Field Testing /Inspection |  | Review Submittals |  | Review Testing, Certification, & Lab Reports |  |
| 1705.6 Soils |  |  |  |  |  |  |  |
| Observe site preparation, fill placement and testing of compaction for compliance with the construction documents for the project |  | A, C, I, R |  |  |  |  |  |
| Observe and test bearing materials below shallow foundations for ability to achieve design bearing capacity |  | A, L |  |  |  |  |  |
| Review compaction testing for compliance with the construction documents for the project |  |  |  |  |  | A |  |
| 1705.7, 1705.8 & 1705.9, 1705.10 Driven Deep, Cast-in-place Deep, and Helical Pile Foundations |  |  |  |  |  |  |  |
| Observe installation |  | A, L, I |  |  |  |  |  |
| Observe load tests |  | A, I |  |  |  |  |  |
| 1705.12 Special Inspection for Wind Resistance |  |  |  |  |  |  |  |
| Structural wood |  | A, N |  |  |  |  |  |
| Cold-Formed steel light-frame construction |  | A, N |  |  |  |  |  |
| Inspect roof cladding |  | A, B, N |  |  |  |  |  |
| Inspect wall cladding |  | A, B, N |  |  |  |  |  |
| 1705.13 Special Inspection for Seismic Resistance |  |  |  |  |  |  |  |
| 1705.13.1 Structural Steel |  |  |  |  |  |  |  |
| Inspection of structural steel in the seismic force-resisting systems |  | A, C |  |  |  |  |  |
| 1705.13.2 Structural Wood |  |  |  |  |  |  |  |
| Inspection of structural wood in the seismic force-resisting systems |  | A, N |  |  |  |  |  |
| 1705.13.3 Cold-Formed Steel Light-Frame Construction |  |  |  |  |  |  |  |
| Inspection of cold-formed steel light-frame construction in the seismic force-resisting systems |  | A, N |  |  |  |  |  |
| 1705.13.4 Designated Seismic Systems |  |  |  |  |  |  |  |
| Examine designated seismic systems requiring seismic qualification and verify that the label, anchorage or mounting conform to the certificate of compliance |  | A |  | A |  | A |  |
| 1705.13.5 Architectural Components |  |  |  |  |  |  |  |
| Inspection of exterior cladding, non-load bearing walls, veneer, and access floors |  | A, B |  | A, B |  | A, B |  |
| 1705.13.6 Plumbing, Mechanical and Electrical Components |  |  |  |  |  |  |  |
| Inspection of installation and anchorage of mechanical and electrical components |  | A |  | A |  | A |  |
| 1705.13.7 Storage Racks |  |  |  |  |  |  |  |
| Inspection of anchorage of storage racks 8 feet or taller |  | A |  |  |  |  |  |
| 1705.13.8 Seismic Isolation Systems |  |  |  |  |  |  |  |
| Inspection of seismic isolation systems in seismically isolated structures | A | A |  |  |  |  |  |
| 1705.13.9 Cold-Formed Steel Special Bolted Moment Frames |  |  |  |  |  |  |  |
| Inspection of cold-formed steel special bolted moment frames |  | A, N |  |  |  |  |  |
| 1705.14 Testing for Seismic Resistance |  |  |  |  |  |  |  |
| Testing designated seismic systems requiring seismic qualification and verify that the label, anchorage or mounting conform to the certificate of compliance |  | A |  |  |  |  |  |
| 1705.15 Sprayed Fire-Resistant Materials |  |  |  |  |  |  |  |
| Observe surface conditions, application, average thickness and density of applied material, and cohesive/adhesive bond |  | A, C |  |  |  |  |  |
| (Table continued on next page) |  |  |  |  |  |  |  |
| MINIMUM SPECIAL INSPECTOR QUALIFICATIONS (continued) |  |  |  |  |  |  |  |
| Category of Testing and Inspection | Minimum Qualifications (refer to key at end of Table) |  |  |  |  |  |  |
|  | Shop Inspection | Field Testing /Inspection |  | Review Submittals |  | Review Testing, Certification, & Lab Reports |  |
| 1705.16 Mastic and intumescent fire-resistant coatings |  |  |  |  |  |  |  |
| Observe application compliance with AWCI 12-B |  | A, C |  |  |  |  |  |
| 1705.17 Exterior Insulation and Finish Systems |  |  |  |  |  |  |  |
| Inspect EIFS systems |  | A, B, C, M |  |  |  |  |  |
| 1705.18 Fire-resistant penetrations and joints |  |  |  |  |  |  |  |
| Inspection of Penetration firestops |  | A, C, P |  |  |  |  |  |
| Inspection of Fire-resistant joint systems |  | A, C, P |  |  |  |  |  |
| 1705.19 Testing for Smoke Control | See Requirements of Building Code Section 1705.19.2. |  |  |  |  |  |  |
| 1705.20 Sealing of Mass Timber |  |  | A, C, P |  |  |  |  |
| (Table continued on next page) |  |  |  |  |  |  |  |

MINIMUM SPECIAL INSPECTOR QUALIFICATIONS (continued)

KEY: - Arkansas Professional Engineer (AR PE) competent in the specific task area or graduate of accredited engineering/engineering technology program under the direct supervision of an AR PE. - Arkansas Registered Architect (AR RA) competent in the specific task area or graduate of accredited architecture/architecture technology program under the direction of an AR RA. - International Code Council (ICC) Special Inspector Certification specific to the particular material and testing methodology applicable to each Category of Testing and Inspection listed in the table. - Post-tensioning Institute (PTI) Certification, Level 2. - Pre-stressed Concrete Institute (PCI) Plant Quality Personnel Certification – Level III. - American Welding Society (AWS) Certified Welding Inspector (CWI) or AWS Certified Associate Welding Inspector working under the direct on-site supervision of a CWI. - American Society for Nondestructive Testing (ASNT) Level II certification, or a Level III certification if previously certified as a Level II in the particular material and testing methodology applicable to each Category of Testing and Inspection listed in the table. - American Concrete Institute (ACI) Concrete Construction Special Inspector. - National Institute for Certification in Engineering Technologies (NICET) Level II or higher certification specific to the particular material and testing methodology applicable to each Category of Testing and Inspection listed in the table. - ACI Concrete Field Testing Technician with Grade 1 certification or Center for Training Transportation Professionals (CTTP) Certified Concrete Field Testing Technician. - American Concrete Institute (ACI) Masonry Field Testing Technician - NICET Certified Engineering Technologist (CT) competent in the specific task area. - Association of the Wall and Ceiling Industry (AWCI) EIFS Inspector Certification. - International Code Council (ICC) Commercial Building Inspector Certification. - International Code Council (ICC) Mechanical Inspector Certification. - Inspector has passed either the Underwriters Laboratory (UL) Firestop Contractor Program Examination or the Factory Mutual (FM) Firestop Examination. - Pre-stressed Concrete Institute (PCI) Certified Field Auditor - Center for Training Transportation Professionals (CTTP) Certified Soil Testing Technician. - American Concrete Institute (ACI) Post-Installed Concrete Anchor Installation Inspector Notes: 1. The Special Inspector shall meet one of the minimum qualifications listed for the applicable Category of Testing and Inspection. 2. Materials testing shall be done by an Approved Testing Agency meeting the requirements of the Building Code Section 1703 and ASTM E 329.